RK Tek  734/827 Big Bore

- Clean ALL new and old parts thoroughly Clean ALL new and old parts thoroughlyto assure no foreign debris is present. After cleaning with solvent/and or Soapy Water. Coat the cylinder walls with 2 stroke oil and then wipe out the oil with a clean rag. This oil will pull even more debris from the plating leave the excess oil

To set end gap: a) Place 1 ring in cylinder	End gap should be .018”-.020”

b) Level ring in cylinder by using the piston as a leveling device. Measure gap with a feeler gauge.

c) Using a wet-stone or something similar, gently grind on one end of the ring until desired

clearance is obtained. Make sure to grind at the same angle as the original.

d) Repeat process for each ring.

e) MAKE SURE you know which rings are gapped for which cylinders!

- Install rings on pistons. Letters on rings always up. End gap goes opposite exhaust port.

- Install pistons and top end bearing onto connecting rods. Ring ends always point to intake.

- Install wrist pin and cir-clips onto piston. MAKE SURE cir-clips are full seated in groove!

- Install, RKT supplied, base gasket. To assure a flat seal. MAKE SURE case and cylinder base is free of ANY grease or foreign debris. Install MAG cylinder. Hand-tighten bolts. Make sure the gasket has not been displaced during cylinder installation. Install PTO cylinder.

- Install RAVE gaskets (not supplied) under each RAVE guillotine. Make sure RAVE base housing has exhaust supply hole line up with supply hole on cylinder. Bleed hole toward belly-pan. Make 100% sure that the exhaust valves have at least .020” from the valve to the piston.

- Torque cylinder bolts to 27 ft/lbs and then final torque bolts to 32ft/lbs.

- Install Head as per supplied instructions.(see online) Torque @ 22ft/lbs

- Do not run the BR9ECS plug run the BR9EYA plugs (Arctic Cat) or BR9ES gap the plug @.020”

- PREMIX a 125:1 oil ratio in 1st tank of fuel. RKT Recommends to always premix at 150:1 in addition to your oil injection system. DO NOT USE AMSOIL EVER!! Break in engine with a mineral based oil and NEVER run ANY synthetic oil in this engine.> Ever!

- IMPORTANT: DO NOT USE ANY NON RK Tek approved aftermarket silencer or pipe.

- Fire engine and check for base gasket leakage. Make sure there are no air pockets in the coolant system.

- Run engine at varying throttle positions for the 1st 70 miles checking piston wash and plug color (and adjusting accordingly)! NOTE: The 827 engines are 8400-8500rpm engines! 734 kit runs at 8000-8200rpm. NOTE: These engines take 100 miles for the rings to fully seat.

- Jetting (734 kit):> on the Flat-Side carbs, start with 380 mains, 20/17.5 pilots and Richen the needle 1 clip.. Round Slide:->> 310 mains, 40 pilots, stock needle. NOTE: These are JUST starting recommendations…FINAL jetting could be much different (usually much lower)

- Jetting (827 kit): If you have hooded needles (SOME ‘03 and newer) start with 400 mains and work down from there (probably end up at 350’s) richen needle 1 clip. For ‘01 and ‘02 , non hooded carbs, start with 480 mains and work down.(usually end up at 440 main)

- Clutching 800HO: Below 3000ft : Pur/Pnk pri spring 414 ramp 17-21g in the pins and set clicker to #3 setting.  If above 3000ft-> 413 or 415 ramp start with 14g pin weight. Run the engine ON SNOW under load and adjust pin weight until the engine is pulling desired rpm..then click up 1 on your primary and you should be there. NOTE: do NOT assume that if you are correct rpm that you have the correct LOAD on the engine. ALWAYS load the engine UNTIL it will no longer pull the correct rpm. Then adjust clickers.