RK Tek Torque-Line KTM/BETA/HUSQVARNA Head

Installation Instructions

- Drain coolant on original head. We recommend removing the water pump drain bolt to do this.  Replace bolt after coolant is drained.

- Remove fuel tank and seat.

- Remove head stay from OEM Head. Removed coolant hose from OEM Head.

- Remove head bolts and washers. These will be reused.

- Remove OEM Head via lifting it off the cylinder. If any coolant is in the cylinder, it needs to be removed before proceeding.

- If the 2 locating dowels remained in the OEM Head, remove them and install in the cylinder.

- Cylinder O-Rings (2) should remain in cylinder. Make sure these are seated properly in their associated groove.

- Install 6 Bolt Sealing O-Rings into the Torque-Line Head Shell in their associated grooves located at each bolt hole on the inside of the head shell.

- Install Dome Sealing O-Ring (1) on billet dome around spark plug hole.

- Keeping the head shell flat, install the dome, making sure that the 6 bolt sealing O-Rings do not get displaced. NOTE: This dome will only install in 1 position into the head shell.

- Place the Billet Head, with dome installed, onto the cylinder. Make sure you do not displace the Cylinder O-Rings or locating dowels.

- Install the 6 head bolts and washers (Step #4). Torque bolts first to 14 ft/lbs and then to 21 ft/lbs using an alternating “cross” pattern.

- Connect coolant hose, securing clamp and re-install the head stay.

- Fill cooling system with a 50/50 anti-freeze/water mix of your choice.

- Remount fuel tank and seat. Start engine and verify there are no leaks anywhere.

- Ride the motorcycle until the engine gets up to operating temperature. Park bike, let cool, and re-check coolant level to be sure it is full.

- Jetting: Proper jetting will vary from bike to bike. There are MANY variables in play that determine the proper carburetor settings for your engine. IF your engine was jetted correctly before the head install, then it should be fine after. We find changing the needle 1 clip richer seems to work well with the head.